Smart packaging applications and their effect on packaging printing? Examining new approaches in the printing industry

Spyridon Nomikos, Anastasios E. Politis*, Jenny S. Darzentas, Thomas Spyrou, John Darzentas
Department of Product and Systems Design Engineering, University of the Aegean, Hermoupolis, 84100 Syros, Greece, 
{nomic; jennyd; tsp; idarz}@aegean. gr.

*Technological Educational Institution-TEI of Athens, Aegaleo, 12210, Athens, Greece{politisresearch@techlink.gr}*

# Abstract: Traditional packaging printing is being challenged by new technologies, in terms of the application of the digital information onto the physical – material based- package. This is due to the  widening range of implementations of electronic processing environments and the processing of not only printed, but also electronic media. The applications of new technologies in the packaging area and, in particular the integration of digital information of every type of graphic or text on the packaging substrate is an emerging issue for furthering the development of packaging printing. The objective of this paper is to explore the processes and workflows of producing the new “hybrid’ packaging by the traditional printing processes. In particular the main printing methods applied in packaging will be examined (Offset, Flexography, Gravure, Screen, Pad printing, Letterpress and Digital printing) correlated with materials, production processes and workflows. Furthermore, this paper will examine the potential of the development and application of new models of operation in the affected industries.

Keywords: Packaging technologies, printing methods, new printing models, smart and hybrid packaging

# Introduction

Packaging is currently at the centre of intensive research activity regarding new technologies that integrate digital data on a package. Various applications and pilot tests are carried out, fuelled by developments in electronics technology, materials and processes. These technologies appear as “intelligent”, “smart” or “hybrid” packaging. The new hybrid packaging incorporates microchips, tags and antennas as well as new materials such as time-thermochromic inks and various indicators tracking changes, real time responses and wireless communication.

These developments in packaging production need to be investigated thoroughly with regard tο their adaptation into traditional packaging printing production. We attempt to carry out this investigation firstly by discussing definitions of intelligent packaging, including labels and tags, with their characteristics, and their applications in products and sectors. Next, the main stages of the graphic arts production are presented, and the potential and constraints of printing methods are highlighted. Finally the developments in integrating smart packaging applications within the prepress, printing and postpress processes of packaging are discussed.

Traditionally, the basic functions of packaging have been classified into four categories, protection, communication, convenience, and containment [1], [2]. The package is used to protect the product against the deteriorative effects of the external environment, to communicate with the consumer as a marketing tool, to provide the consumer with greater ease of use and time-saving convenience, and to contain products of various shapes and sizes. Nor are these functions totally exclusive; for example, the communication function of the package through warning labels and cooking instructions can also help to enhance food protection and convenience[3].

Packaging is one of the most dynamic sectors in the printing industry, and our research begins by trying to define the new environment, where, with the incorporation of chip, antenna, and battery, packaging is transformed from inanimate static packaging into packaging, with communicative abilities and dynamic behaviour. The aim of our study is to investigate and capture the current situation from the environment of conventional technology for printed communication, through to the packaging sector, with a view to optimising the collaboration between the conventional and digitally produced information. Such collaboration will help to structure the emerging environment, where print and electronic elements will coexist on the physical substrate of the package. thus transforming the static optical message into dynamic media………………………………………….

…………………………………………………………………

TABLE 2

| METHOD OF PRINT MEDIA | DEGREE OF FACILITY OF TREATMENT DURING PRE PRESS | DEGREE OF SUITABILITY OF PRINTING PLATE- UTERUS | DEGREE OF TECHNOLOGICAL INFRASTRUCTURE FOR THE PRINTING OF ANTENNA | DEGREE OF INCORPORATION RFID | MATERIALS FOR PACKAGING BOX |
| --- | --- | --- | --- | --- | --- |

# References

1. Paine FA. 1991. The packaging user’s handbook. New York: AVI, Van Nostrand Reinhold. 158 p.

2. Robertson GL. 1993. Food packaging: principles and practice. Marcel Dekker, New York. 686 p.

3. Kit L. Yam, Paul T. Takhistov, Joseph Miltz, Journal of Food Packaging, Vol. 70, Nr. 1, 2005

4. Raghu Das,  Peter Harrop, Glyn Holland (2005), The A to Z of printed and disposable Electronic, IDTechEx, Cambridge, UK.

5. MIT, Auto-Id Labs, 6-8 April 2004, Presentations

6. IDtechEX 19-20/4/2005 Cambridge, reports in Smart Packaging.

7. Baumann, R. & Weiss R. , (2005) Industrial Printing Opportunities for Polymer Electronics”, MAN Roland Druckmaschinen AG, in the Printed Electronics Europe conf. , Cambridge, UK, (IDTechEx).

8. Spyridon Nomikos, Anastasios E. Politis, Jenny Darzentas, Thomas Spyrou, John Darzentas, Proceeding, Exploring cross-media concepts for future packaging – Challenges for the printing industry/ s, 32nd inter. Conf. Iarigai. 4-7 sept 2005, Digitalisation and print media p. p. 317-319

9. John Cooney-Barry Winkless, University college cork, Ireland (next generation of food packaging concepts)

10.	package printing 2006 http://www. packageprinting. com/doc/ppprepressforpackagingsic. bsp (february18, 2006)

11.	[Xink Labs Ltd. 2150 Thurston Drive, Suite 101Ottawa, Ontario, Canada. "High-speed printing is the most feasible option to supply RFID tags in the high billions" September 20, 2005

12.	Bruce E. Kahn, (presentations)Wisconsin RFID Conference, Waukesha, WI, June 22, 2005

13.	PolyApply Luigi Occhipinti, PolyApply Integrated Project The 1st and 2nd PolyApply Newsletter (May 2004, Nov. 2004) (www. polyapply. org)

14.	ACREO AB, Tommi Remonen,, Presented at Printed Electronics Europe 2005, Cambridge, UK, April 25, 2005

15.	Stefan Mieslinger, Project Manager Smart Label. [Editing By Converting Magazine Editor in Chief Mark Spaulding.

16.	OMRON Technology to produce RFID tags, June 02 2005, OMRON Corporation Wakahiro Kawai.

17.	 Pad printing. The Wiley Encyclopedia of packaging technology /Marilyn Bakker/John Wiley & Sons/p. p. 242-243.

18.	Christian Menegon, Category Manager, Industrial Printing Presses HP Indigo, U. S. Presented at Printed Electronics Europe 2005, Cambridge, UK, April 25, 2005.

20.	Peter Harrop An Introduction to Smart Packaging IDTechEx 19-20/April 2005.

21. The Microsoft Perspective (Submitted to the Federal Trade Commission)as a follow-on to the Workshop on RFID: Applications and Implications for Consumers, June 21, 2004 / Authors :Kim Hargraves, Senior Privacy Strategist, Steven Shafer, Senior Researcher.

22. Lindner Thomas, Printed Electronics Cambridge, UK, April 25 2005–HP- Technology beyond, Ink on Paper Hewlett Packard Co.

23. Fairley Mike. The role for the label, printing and converting industry in Smart Labels, Director, Strategic Development, Labels Group, Τarsus, U. K.

24.	PRECISIA 2004.	, Rethinking Print, Printed Antenna, 2004 Precisia, LLC. Delo Industrial Adhesives, Germany (Smart Labels USA 2005 on Jun 29, 2005).

25.	Permalink, RFID Comments /January 25,/2006, RFID Gazette.

26.[Bielomatik], www.Bielomatik.com, / RFID Functional /The Qualified Manufacturing Process-

Author Biography

Spyridon Nomikos received ,BA-in Technology of Graphic Arts in TEI of Athens and BSc from University of Middlesex in 2001.Since then he has worked in research area of Smart Packaging in University of the Aegean, Department .of Product and Systems Design Engineering. He is Lecture in Technological Educational Institutions (TEI) -Graphic Arts Technology. Member of TAGA and GATF.